JACKHAMMER Baseboard and Buildup Stripper

MATERIAL SAFETY DATA SHEET

Version Number: 1

Preparation date: 2008-03-06

1.   PRODUCT AND COMPANY IDENTIFICATION

Product name:	JACKHAMMER Baseboard and Buildup Stripper

MSDS #:	MS0400131

Product Code:	4966777

Recommended use:	Industrial/Institutional. Floor care.

Manufacturer, importer, supplier:

US Headquarters

The Butcher Company

8310 16th St.

Sturtevant, Wisconsin 53177-0902

Phone: 1-800-225-9475

MSDS Internet Address:

www.thebutchercompany.com

Emergency telephone number:	1-800-228-5635 (Prosar); 1-651-917-6133 (Int'l Prosar); 01-800-710-3400 (México)

2. HAZARDS IDENTIFICATION

EMERGENCY OVERVIEW

•   DANGER

•   CORROSIVE

•   CAUSES SKIN AND EYE BURNS

•   HARMFUL OR FATAL IF SWALLOWED

Principle routes of  exposure:	Eye contact. Skin contact. Inhalation.

Eye contact:	Corrosive. Causes permanent eye damage, including blindness.

Skin contact:	Corrosive. Causes permanent damage.

Inhalation:	May cause irritation and corrosive effects to nose, throat and respiratory tract.

Ingestion:	Corrosive. Causes burns to mouth, throat and stomach.

3. COMPOSITION/INFORMATION ON INGREDIENTS

Hazardous ingredients

| Ingredient(s) | CAS # | Weight % |
| --- | --- | --- |
| Benzyl alcohol | 100-51-6 | 0.1 - 1.5% |
| 2-butoxyethanol | 111-76-2 | 1 - 5% |
| Diethylene glycol butyl ether | 112-34-5 | 1 - 5% |
| Potassium hydroxide | 1310-58-3 | 1 - 5% |
| Monoethanolamine | 141-43-5 | 1 - 5% |
| Isobutane | 75-28-5 | 3 - 7% |

4. FIRST AID MEASURES

Eye contact:	Immediately flush eyes with running water for 15-20 minutes, keeping eyelids open. Get medical attention immediately.

Skin contact:	Immediately flush with plenty of water for 15-20 minutes . Get medical attention immediately.

Inhalation:	If breathing is affected, remove to fresh air. Get medical attention immediately.

Ingestion:	If swallowed, rinse mouth. Give a cupful of water or milk . THEN IMMEDIATELY CONTACT A PHYSICIAN OR POISON CENTER. DO NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person.

Aggravated Medical Conditions:	Individuals with chronic respiratory disorders such as asthma, chronic bronchitis, emphysema, etc., may be more susceptible to irritating effects.

JACKHAMMER Baseboard and Buildup Stripper

1 of 5

5. FIRE-FIGHTING MEASURES

Suitable extinguishing media:

Use dry chemical, CO2, water spray or "alcohol" foam.

Extinguishing media which must not be used for safety reasons:

No information available

Special protective equipment for firefighters:

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear

Specific hazards: Aerosol product - Containers may rocket or explode in heat of fire. NFPA 30B Level 1 Aerosol  .

Unusual hazards: Corrosive material (See sections 8 and 10).

Specific methods: Aerosol Product - Containers may rocket or expode in heat of fire. Use water spray to keep fire-exposed containers cool.

NFPA

Health: 3	Flammability: 4	Instability:  0

6. ACCIDENTAL RELEASE MEASURES

Personal precautions:	Put on appropriate personal protective equipment (see Section 8.).

Environmental precautions and clean-up methods:

No information available

7. HANDLING AND STORAGE

Handling:	Avoid contact with skin, eyes and clothing. Do not taste or swallow. Avoid breathing vapors or mists.

Use only with adequate ventilation. Remove and wash contaminated clothing and footwear before re-

use. Wash thoroughly after handling. Product residue may remain on/in empty containers. All precautions for handling the product must be used in handling the empty container and residue. FOR COMMERCIAL AND INDUSTRIAL USE ONLY.

Storage:	Protect from freezing. Keep tightly closed in a dry, cool and well-ventilated place. KEEP OUT OF REACH OF CHILDREN.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

Engineering measures to reduce exposure:

Good general ventilation should be sufficient to control airborne levels. Respiratory protection is not required if good ventilation is maintained.

Personal Protective Equipment

Eye protection:	Chemical-splash goggles.

Hand protection:	Chemical-resistant gloves

Skin and body protection:	Protective footwear. If major exposure is possible, wear suitable protective clothing and footwear.

Respiratory protection:	In case of insufficient ventilation wear suitable respiratory equipment. A respiratory protection program that meets OSHA's 29 CFR 1910.134 and ANSI Z88.2 requirements must be followed whenever workplace conditions warrant a respirator's use.

Hygiene measures:	Handle in accordance with good industrial hygiene and safety practice

| Ingredient(s) | ACGIH | OSHA | Mexico |
| --- | --- | --- | --- |
| 2-butoxyethanol | 20 ppm (TWA) | Skin 240 mg/m3 (TWA) 50 ppm (TWA) | 75 ppm (STEL) 360 mg/m3 (STEL) 26 ppm (TWA) 120 mg/m3 (TWA) |
| Potassium hydroxide | 2 mg/m3 (Ceiling) |  |  |
| Monoethanolamine | 6 ppm (STEL) 3 ppm (TWA) | 3 ppm (TWA) 6 mg/m3 (TWA) | 6 ppm (STEL) 15 mg/m3 (STEL) 3 ppm (TWA) 8 mg/m3 (TWA) |
| Isobutane | 1000 ppm (TWA) |  |  |

HMIS

Health	3	Fire Hazard	4	Reactivity	0

Personal protective equipment

9. PHYSICAL AND CHEMICAL PROPERTIES

Physical State: Aerosol

Color: Clear White

Appearance: Aerosol

Odor:  Solvent

pH: 13.0	Dilution pH: No information available.

Specific gravity: 0.99	Density:  8.24  lbs/gal

Bulk density: No information available	Vapor density: No information available

Evaporation Rate  No information available

Solubility: completely soluble

Solubility in other solvents: No information available

Partition coefficient (n-octanol/water): No information available

Viscosity: No information available

VOC: 10.75% *

| Boiling point/range: | Not determined |
| --- | --- |
| Melting point/range: | Not determined |
| Flash point: | <20 (Propellant)°F <-7°C |
| Decomposition temperature: | Not determined |
| Autoignition temperature: | No information available |
| Elemental Phosphorus: | 0.11 %P |

* - Title 17, California Code of Regulations, Division 3, Chapter 1, Subchapter 8.5, Article 2, Consumer Products, Sections 94508

10. STABILITY AND REACTIVITY

Stability:	The product is stable

Polymerization:	Hazardous polymerization does not occur

Materials to avoid:	Acids. Metals.

Conditions to avoid:	Keep away from heat. Keep away from open flames, hot surfaces and sources of ignition.

Hazardous decomposition products:	None reasonably foreseeable.

11. TOXICOLOGICAL INFORMATION

Hazardous ingredients

| Ingredient(s) | LD50 Oral - Rat (mg/kg) | LD50 Dermal - Rabbit | LC50 Inhalation - Rat |
| --- | --- | --- | --- |
| Benzyl alcohol | 1230 | 2000 mg/kg | 8.8 mg/L (4 h) |
| 2-butoxyethanol | 470 | 220 mg/kg | 2.21 mg/L (4 h); 450 ppm (4 h) |
| Diethylene glycol butyl ether | 4500 | =2700 mg/kg | Not available |
| Potassium hydroxide | 275 | Not available | Not available |
| Monoethanolamine | 1720 | 1 mL/kg | Not available |
| Isobutane | Not available | Not available | =658 mg/L (4 h) |

Acute toxicity:	Corrosive Oral LD50 estimated to be between 2000 - 3000 mg/kg Dermal LD50 estimated to be > 2000 mg/kg

Chronic toxicity:	None known

Specific effects

Carcinogenic effects:                        None known Mutagenic effects:                             None known Reproductive toxicity:                      None known Target organ effects:                         None known

12. ECOLOGICAL INFORMATION

12. ECOLOGICAL INFORMATION

Environmental Information:	No data available

13. DISPOSAL CONSIDERATIONS

Waste from residues / unused products:

Dispose of according to all federal, state and local applicable regulations

14. TRANSPORT INFORMATION

DOT/TDG: Please refer to the Bill of Lading/receiving documents for up to date shipping information

15. REGULATORY INFORMATION

International Inventories

All components of this product are listed on the following inventories: U.S.A. (TSCA), Canada (DSL/NDSL).

U.S. Regulations

California Proposition 65: This product is not subject to the reporting requirements under California's Proposition 65

STATE RIGHT TO KNOW

| Ingredient(s) | CAS # | MARTK: | NJRTK: | PARTK: | RIRTK: |
| --- | --- | --- | --- | --- | --- |
| Benzyl alcohol | 100-51-6 | X | X | X | - |
| 2-butoxyethanol | 111-76-2 | X | X | X | X |
| Diethylene glycol butyl ether | 112-34-5 | - | X | X | - |
| Potassium hydroxide | 1310-58-3 | X | X | X | X |
| Monoethanolamine | 141-43-5 | X | X | X | X |
| Water | 7732-18-5 | - | - | - | - |
| Ethoxylated secondary alcohol | 84133-50-6 | - | - | - | - |
| Isobutane | 75-28-5 | X | X | X | - |

CERCLA/ SARA

| Ingredient(s) | Weight % | CERCLA/SARA RQ (lbs) | Section 302 TPQ (lbs) | Section 313 |
| --- | --- | --- | --- | --- |
| 2-butoxyethanol 111-76-2 | 1 - 5% |  |  | X |
| Diethylene glycol butyl ether 112-34-5 | 1 - 5% |  |  | X |
| Potassium hydroxide 1310-58-3 | 1 - 5% | 1000 |  |  |

| Ingredient(s) | CAA HAP | CAA ODS | CWA Priority Pollutants |
| --- | --- | --- | --- |
| Diethylene glycol butyl ether 112-34-5 | X |  |  |

SARA 311/312 Hazard Categories Immediate:	X Delayed:	- Fire:	X Reactivity:	- Sudden Release of Pressure:	X

Canada

WHMIS hazard class: E    Corrosive material , A   Compressed gases.

16. OTHER INFORMATION

Reason for revision:	Not applicable

Prepared by:	NAPRAC

Additional advice:	None

Notice to Reader: This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained within. Actual conditions of use and handling are beyond seller's control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State, Provincial and Local laws and regulations.